# PICO AUDIO I2S

There are several factors that are key to producing high quality audio with the Raspberry Pi Pico.

- Clocks with accurate frequencies, phase relationships and low jitter.

- Good board layout.

- Input and output filtering.

While many tutorials on Pico I2S focus only on output through a DAC or I2S amplifier IC, here we will also deal with signal input using audio CODECs.

# Clocks

Design constraints:

- Accurate clock frequencies ensure that the pitch of the captured or produced sound are as expected.

- Clock jitter must be minimised, and hopefully eliminated, to minimise distortion of the audio signal.

- Correct phase relationships between the various clock signals, namely MCLK, BCLK (SCLK) and LRCLK (WCLK) must be maintained.

- Some DACs and CODECs do not require MCLK, as they contain internal PLLs that can generate the signal.

- Other system functions (e.g. USB) must not be disturbed by adjusting the RP2040 clocking to generate I2S signals.

## Jitter

Jitter needs to be addressed at two levels – the RP2040 sys_clk frequency needs to have the lowest possible jitter, as well as the MCLK and BCLK signals created by the PIO clock dividers and code.

[sys_clk] Jitter is minimised by running the VCO at the highest possible frequency, so that higher post-divide values can be used. (DS 230)

For the PIO clock dividers it is best that the integer divisor be an even number and the fractional part should be zero, as other values will introduce unacceptable clock jitter from the delta-sigma fractional divider, particularly at low fractional values.

??? Should the sys_clk and PIO divisors also be a power of 2????

The [PIO] clock dividers are 16-bit integer, 8-bit fractional, with first-order delta-sigma for the fractional divider. The clock divisor can vary between 1 and 65536, in increments of 1/256. …

For small n, the jitter introduced by a fractional divider may be unacceptable. However, for larger values, this effect is much less apparent (DS 338)

For the lowest possible jitter, as many of the dividers as possible should have divisors that are powers of two (1, 2, 4,8, etc). This applies most importantly to dividers close to the signal output, as jitter from dividers further up the chain tends to be evened out by the following dividers.

Ultimately, a balance needs to be struck between reducing jitter and finding achievable VCO and sys_clk frequencies.

## Frequency

Finding an appropriate sys_clk frequency for each of the required MCLK frequencies is not a trivial exercise given all the constraints, outlined above.

To avoid clock jitter

- sys_clk to PIO clock divisors for MCLK / BCLK should be integral and even numbers. 
As MCLK is always an even, integral multiplier of BCLK, constraining MCLK is sufficient to ensure an appropriate BCLK divisor.

- While sys_clk should preferably be an even (or, even better, a power of 2) divisor of the VCO frequency (particularly the POSTDIV2 value), this may prove to be an unachievable constraint.

FOUTPOSTDIV can then be calculated as (FREF / REFDIV) × FBDIV / (POSTDIV1 × POSTDIV2). With a desired output frequency in mind, you must select PLL parameters according to the following constraints of the PLL design:

• Minimum reference frequency (FREF / REFDIV) is 5MHz

• Oscillator frequency (FOUTVCO) must be in the range 750MHz → 1600MHz

• Feedback divider (FBDIV) must be in the range 16 → 320

• The post dividers POSTDIV1 and POSTDIV2 must be in the range 1 → 7

• Maximum input frequency (FREF / REFDIV) is VCO frequency divided by 16, due to minimum feedback divisor Additionally, the maximum frequencies of the chip’s clock generators (attached to FOUTPOSTDIV) must be respected. For the system PLL this is 133MHz, and for the USB PLL, 48MHz.

(DS 229)

A scan of several common DACs and CODECs (see Appendix A) produced a common denominator of the following MCLK multipliers and frequencies. Some devices will only accept one of the MCLK multipliers for each word length.

The PIO divider will need to create a frequency twice the desired MCLK, as there are two clock states to be programmed.

| Common MCLK frequencies (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Word length | MCLK multiplier | MCLK |  | Double MCLK |  |  |
|  |  | 44.1 kHz | 48 kHz | 44.1 kHz | 48 kHz |
| 16 | * 128 fs | 5.6448 | 6.144 | 11.2896 | 12.288 |
|  | 256 fs | 11.2896 | 12.288 | 22.5792 | 24.576 |
|  | 512 fs | 22.5792 | 24.576 | 45.1584 | 49.152 |
| 24 | 192 fs | 8.4672 | 9.216 | 16.9344 | 18.432 |
|  | 384 fs | 16.9344 | 18.432 | 33.8688 | 36.864 |

* not supported by all CODECs and DACs

Practically, the following sys_clk values provide results close to the required Fs.

| ### Suitable system clocks for I2S audio |  |  |
| --- | --- | --- |
| ### * 48 kHz | ### sys_clk | ### 147.6 Mhz |
|  | ### Error | ### -0.098% |
| ### †44.1 kHz | ### sys_clk | ### 135.6 Mhz |
|  | ### Error | ### -0.092% |

*  8, 16, 48 kHz all word lengths
† all word lengths

| PIO divisors |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Word length | MCLK multiplier | 48kHz and 44.1kHz |  | 16kHz |  | 8kHz |  |
|  |  | MCLK PIO divisor | BCLK PIO divisor | MCLK PIO divisor | BCLK PIO divisor | MCLK PIO divisor | BCLK PIO divisor |
| 16 | 128 fs | 12 | 384 | 36 | 1152 | 72 | 2304 |
|  | 256 fs | 6 | 768 | 18 | 2304 | 36 | 4608 |
|  | 512 fs | 3 | 1536 | 9 | 4608 | 18 | 9216 |
| 24 | 192 fs | 8 | 384 | 24 | 1152 | 48 | 2304 |
|  | 384 fs | 4 | 768 | 12 | 2304 | 24 | 4608 |

## Phase

The phase between LRCLK and BCLK is critical of all surveyed CODECs and DACs, and is managed directly within a single PIO state machine.

MCLK to BCLK phase is not critical, but should not shift over time. This is achieved by having related PIO clocks for the LRCLK/BCLK and MCLK state machines.

## PIO

# References

DS	RP2040 Datasheet

SDK	Raspberry Pi Pico C/C++ SDK

QQ	Quihong Qu, Delta-Sigma Fractional-N Synthesizers Enable Low-Cost, Low-Power, Frequency-Agile Software Radio, https://www.eetimes.com/delta-sigma-fractional-n-synthesizers-enable-low-cost-low-power-frequency-agile-software-radio/

JW	Jim Wright, Conexant, Enabling next-generation wireless applications, Using delta-sigma fractional-n synthesizers reduces the overall bill-of-materials cost and provides for cost-effective solutions in both base stations and terminals for 2.5- and 3-G networks, https://web.archive.org/web/20070825195525/http://www.electronicproducts.com/ShowPage.asp?SECTION=3700&PRIMID=&FileName=FEBCON1.FEB2001

# Appendix A

Assessments have only been made for

- Sample frequency (fs) 		8kHz, 16kHz, 44.1kHz and 48kHz.

- Word length 			16, 24 bits

Other sample rates and word lengths have not been assessed or accommodated as they are not common in practical audio applications. Double and quad speed modes have not been assessed as they require significantly higher clock speeds and overall data rates.

There are two common I2S serial audio synchronisation modes:

STD	Phillips or I2S standard. The first BCLK cycle after LRCLK rises is not sampled.

LJ	No delay between LRCK rising and first sampled BCLK

Both modes are synchronised with BCLK falling as LRCLK rises, and data is sampled on the BCLK rising edge.

## DACs and I2S amplifiers

### PCM5102A

| Signal | Constraint |
| --- | --- |
| MCLK input (p 24) | 16 bits: 128fs, 256fs, 512fs 24 bits: 192fs, 384fs |
| PLL Mode - no MCLK (p 24) | 16 bits: 128fs 24 bits: 192fs |
| MCLK Phase (p15) | does not need a specific phase relation between LRCK and system clock [MCLK]. |
| LRCLK / SCK (BCLK) (p15) | If the relationship between LRCK and system clock changes more than ±5 SCK … analog outputs are forced to the bipolar zero until resynchronization between LRCK and system clock is completed. |
| Data formats (p15) | 16 (BCLK = 32fs) or24 bits (BCLK = 48fs) – LJ/STD. 16-bit data packed into 24 or 32-bit slots (BCLK = 48, 64fs) – LJ/STD. 24-bit data packed into 32-bit slots (BCLK = 64fs) – LJ/STD. |

## CODECs

### AK4621

| Signal | Constraint |
| --- | --- |
| MCLK input (Serial mode – p19) (Parallel mode – p20) | 16 bits: 256fs, 512fs 24 bits: 384fs, 768fs |
| Auto setting mode (p19) | 16 bits: 512fs 24 bits: 768fs |
| PLL Mode - no MCLK (p19, 20) | N/A |
| MCLK Phase (p19) | MCLK must be synchronized with LRCK but the phase is not critical. |
| LRCLK / SCK (BCLK) (p15) | AK4621 is reset automatically when the synchronization is out of phase by changing the clock frequencies. |
| Data formats (p21) | 16/24bit LJ 24 bit STD 16-bit data packed into 24 or 32-bit slots (BCLK = 48, 64fs) – LJ. 24-bit data packed into 32-bit slots (BCLK = 64fs) – LJ/STD. |

### SGTL5000

| Signal | Constraint |
| --- | --- |
| MCLK input (p14) | 16 bits: 256fs, 512fs 24 bits: 384fs |
| Auto setting mode | No |
| PLL Mode - no MCLK | Yes |
| MCLK Phase (p14) | MCLK does not need be synchronized with BCLK/LRCK. |
| Synchronization (p22) | DAC and ADC outputs are forced to zero if the synchronization is lost. |
| Data formats (p21) | I2S, LJ, RJ – 16 or 24 bits |

### TLV320AIC series (-3104 datasheet references)

| Signal | Constraint |
| --- | --- |
| MCLK input (p25) | Flexible clock generator |
| Auto setting mode | No |
| PLL Mode | Yes |
| MCLK Phase (p) | MCLK does not need be synchronized with BCLK/LRCK. |
| Synchronization (p | DAC and ADC outputs are forced to zero if the synchronization is lost. |
| Data formats (p) | I2S, LJ, RJ – 16 or 24 bits |

### PCM3060/PCM3070

| Signal | Constraint |
| --- | --- |
| MCLK input (p15) | 16 bits: 256fs, 512fs 24 bits: 384fs, 768fs |
| Auto setting mode | No |
| PLL Mode - no MCLK | PCM3070 only |
| MCLK Phase (p22) | MCLK must be synchronized with LRCK but the phase is not critical. |
| Synchronization (p22) | DAC and ADC outputs are forced to zero if the synchronization is lost. |
| Data formats (p21) | I2S, LJ, RJ – 16 or 24 bits |